Kampus Owl — Demo Guide

How to set up, start, run and recover the talking owl during a demo.

| Part | What it is |
| --- | --- |
| Brain | Gemini 3.8 Live (hears and thinks) |
| Voice | ElevenLabs (speaks through the JBL Go 3) |
| Mic | Logitech Brio 100 webcam |
| Eyes | Two round screens on the Raspberry Pi 5 |

# 1. The day before (10 min)

- Charge the JBL Go 3 and the iPhone (the hotspot drains the phone fast).

- Rehearse once with Section 3 below, in a room like the demo room.

- Check ElevenLabs credit. The free plan is about 20 minutes of owl speech per month; see what's left at elevenlabs.io (bottom-left of the app). The owl prints “ElevenLabs characters used this run” after each reply.

- Bring the Pi's official 27W USB-C power supply, the laptop, and a phone charging cable.

# 2. Set up at the venue (5 min)

- iPhone: Settings → Personal Hotspot → “Allow Others to Join” ON, “Maximize Compatibility” ON. Keep that screen open until step 4 works.

- Pi: plug in the power supply and wait 1 minute. It boots by itself (no power button).

- JBL: turn it on.

- Laptop: connect Wi-Fi to kampus-hotspot (not eduroam), open PowerShell and log in:

ssh hienle@172.20.10.8

If that doesn't answer, use ssh hienle@hienle.local.

# 3. Start the demo (one command)

cd ~/kampus/owl-pi

./run_owl.sh

The launcher checks everything first and prints [ok] for each item: API keys, internet, JBL (it reconnects the speaker and sets the volume), webcam mic, power and temperature.

If a line says [FAIL]: do what it says, then run ./run_owl.sh again.

Then:

- “Calibrating background noise, stay quiet for 2 s” → stay quiet.

- “Sleeping. Say ‘hey owl’…” → the eyes are closed. Ready.

# 4. How to talk to the owl

- Face the webcam and say clearly “Hey Owl” (or “Hello Owl”). The eyes open and it says “Hoo! I'm awake.”

- Ask one question, then stop talking. The eyes look surprised while it hears you, then “thinking”. It starts answering about 2–3 seconds after you stop, and the eyes show an expression (happy, surprised, explaining…).

- Wait until it finishes and the eyes blink once. The blink means “your turn”: the mic is back on. The owl can't hear you while it talks, so it can't be interrupted.

- Say “goodbye” to put it to sleep (it also sleeps after 5 minutes of silence). Say “Hey Owl” to wake it again; each wake starts a fresh conversation.

## Good demo questions

- “Who are you?”

- “Tell me a fun fact about owls.”

- “Can you explain what a Raspberry Pi is, in one sentence?”

- “Give me a tip for studying for exams.”

- “Tell me a joke.”

Tip: keep questions short and one at a time. Long rambling questions, or two people talking at once, confuse it.

# 5. Backup modes

| If this happens | Press Ctrl+C, then run |
| --- | --- |
| “Hey Owl” isn't recognized (loud room) | ./run_owl.sh --no-wake Starts awake; no wake phrase needed. |
| The room is too loud for the mic | ./run_owl.sh --text Type questions on the laptop; the owl still speaks and shows its eyes. |
| No internet at all | .venv/bin/python main.py --demo Eye animations only. |

# 6. Troubleshooting

| Problem | Fix |
| --- | --- |
| ssh doesn't connect | Is the laptop on kampus-hotspot? Is the hotspot screen open on the iPhone? Has the Pi been on for 1 minute? |
| [FAIL] JBL not connected | Press the JBL power button, then run ./run_owl.sh again. (The JBL turns itself off when idle.) |
| [FAIL] no internet | The hotspot is off or the phone locked: open the Personal Hotspot screen again. |
| Owl answers but there's no sound | Turn up the JBL with its + button. |
| Owl doesn't react when you talk | Speak louder and closer to the webcam; wait for the blink before talking. |
| Owl reacts to its own voice | Move the JBL farther from the webcam (at least 30 cm, not facing it). |
| “Cloud request failed” | Usually the hotspot dropped. Check the phone, then just ask again. |
| ElevenLabs quota used up | The month's free credit is gone: use another ElevenLabs account key, or upgrade. |
| Eyes frozen or program crashed | Ctrl+C, then ./run_owl.sh again. |
| Pi unplugged by accident | Follow RESTART_AFTER_POWER_LOSS.txt. |

# 7. After the demo

Ctrl+C stops the owl. Always shut down before unplugging:

sudo shutdown -h now

Wait for the green LED to stop blinking, then unplug.

# 8. Reference

| What | Where |
| --- | --- |
| API keys (Pi only, never in chat or GitHub) | nano ~/kampus/owl-pi/.env |
| Settings: voice, model, volume | ~/kampus/owl-pi/config.toml |
| Brain option in config.toml [pipeline] | brain = "gemini-live" (demo). Also "gemini" or "openai". |
| Pi login | ssh hienle@172.20.10.8 on kampus-hotspot |